F1.1.4 Additional Flights in Open Internationals

In the specification of each outdoor free flight class a procedure is defined for additional flights to decide the individual placings when there is a tie. The maximum flight time is increased for each additional flight subject to conditions. This procedure must be followed at Championships and should be followed at Open Internationals.

At Open Internationals the organisers sometimes have a problem completing this regular procedure. For exceptional reasons of strong winds, poor visibility, inadequate field space, or unavailability of the field for continuation on the following day, Open Internationals may use a non-standard additional flight procedure for all outdoor F1 classes except F1E with the following conditions:

a) A non-standard procedure must be used ONLY for these exceptional reasons of strong winds, poor visibility, inadequate field space, or unavailability of the field for continuation on the following day.

b) An “altitude flyoff” may be specified when F1 altimeters have been approved by CIAM EDIC:

i) The procedures for a regular additional flight for the class are followed

ii) A maximum flight time is defined which should be at least two minutes.

iii) The flight is timed up to the maximum time

iv) For all competitors attaining the maximum flight time, the altitude of the model at the maximum flight time is read from the altimeter and for scoring purposes this value is rounded to the nearest metre.

v) The individual placings are determined by the highest altitudes for all flights attaining the maximum, followed by time order.

vi) Equal altitudes are considered to be a tie, which may be resolved by another additional flight.

Reasons:

1. Beginning of a flight

No altimeter can one-to-one and onto determine the beginning of an official contest-flight of an F1A glider. Altimeters can determine a peak in altitude which is not inevitable the “the release of the model from the launching cable”.

Beginning of timing is defined in

3.1.9 Timing

a) See F1.2.

b) The timing of flights is limited to the maximum durations specified in 3.1.7 and 3.1.8. The total flight time is taken from the release of the model from the launching cable to the end of the flight.

and “F1.1.4 Additional Flights in Open Internationals” is in collision with this rule!

2. Mistake in measurement

“F1.1.4 Additional Flights in Open Internationals” is not defining any applicable certifying procedure which includes:

- measurement error:	e.g.: +/- 0,5m

- quality standard:	1st test:	20 pieces

yearly anonymized test of min. 10 pieces

3. Incomplete rule/procedure to use altimeters

“F1.1.4 Additional Flights in Open Internationals” is not defining any complete procedure to use certified altimeters. Because no measurement error is defined a tie is a lottery. Timekeepers were instructed by the manufacturer of “ALL-TEE” to a procedure which is not regulated in “SECTION 4C – MODEL AIRCRAFT – F1 – FREE FLIGHT”.

Supporting Data:

- standard models without zoom or bunt launch produce no peak in pressure at the moment of releasing the starting cable => the start point can not be defined by altimeter

- damaged high-end models needs to be launched like standard Model  => launch produces no peak in pressure at the moment of releasing the starting cable => the start point can not be defined by altimeter

this happened more than 1 time at WCH and ECH

- starting line breaks and model makes the launch and the flight as normal, but the line and flag are still connected to the hook  =>  altimeter recognize a normal flight what is 100% wrong

- starting line falls off 10 s after launch (this happens some time) => the start point can not be defined by altimeter

There is only 1 altimeter certified altimeter available at the moment. The technical requirement for certification is based on low level requirements which are not adequate! The basic result by using the device is altitude. No measurement error for altitude is defined in “SECTION 4C – MODEL AIRCRAFT – F1 – FREE FLIGHT”

The accuracy for ALL-TEE is announced by the producer/manufacturer as followed:

Minimum relative altitude accuracy:	+/- 0,3	%

Resolution:	10 cm

The accuracy given by the used sensor is given as followed:

relative accuracy over pressure 	(800-1.000 hPa, 25°C):	+/- 0,025 hPa

absolute accuracy over temperature	(800 - 1.000 hPa at 25 - 65°C):	+/- 0,5 hPa

(260 - 1.260 hPa at 00 – 65°C):	+/- 1,0 hPa

Air-pressure over height changes as shown in the table below:

| ALTITUDE | RELATIVE AIR-PRESSURE | AIRPRESSURE |
| --- | --- | --- |
| 000 m | 100,0 % | 1.013,25 hPa |
| 100 m | 098,7 % | 1.000,00 hPa |
| 200 m | 097,5 % | 0.988,00 hPa |

Based on this data the accuracy from produced by the sensor can be calculated:

RELATIVE ACCURACY OVER PRESSURE [RAP]

measuring range: 0 – 100 m                       =>	change in air-pressure = 13,25 hPa

rule of three:	100 m / 13,25 hPa  =  X m / 0,025 hPa

=>	RAP	=	+/- 0,19 m

RELATIVE ACCURACY OVER TEMPERATURE [RAT]

measuring range: 0 – 100 m                       =>	change in air-pressure = 13,25 hPa

rule of three:	100 m / 13,25 hPa  =  X m / 0,5 hPa

=>	RAP	=	+/- 1,89 m

So the RELATIVE ACCURACY from the SENSOR [RAS] is:

RAS   =   RAP + RAT   =   +/- 1,91 m

This calculation is only based on data given by physics and the producer of the sensor. Other parameters like Pressure Accuracy (long-term stability) or Soldering Drift are not recognized at all!

Such a huge mistake in measuring cannot be the basis to create a winner!

How can a device using this sensor produce a much lower mistake in measurement?

Technical Secretary’s Note:

The below references (manuals) have not been included in this Annex for reasons of file size. Any person desiring access to these technical manuals should contact the writer of this proposal via the Austrian Aero Club at modellflug@aeroclub.at

ANNEX1:	data sheet pressure sensor	dm00594286-1799433  -  marked.pdf

ANNEX2:	manual ALL-TEE	All-Tee manual.pdf

ANNEX3:	certificate ALL-TEE	CERTIFICATE - alt001_all-tee_1.0.pdf